Lab 2 — Create a Terrain and Building Pad

Autodesk Revit Architecture Masterclass (C508) · Topic 01: Design Modeling in Revit · Tertiary Infotech Academy Pte Ltd

Goal

Add a toposurface to the building site by placing points at different elevations, then cut a building pad into the terrain based on the footprint of the foundation walls.

You'll build

A landscaped site with a graded toposurface, a grass material and a building pad ready for the model.

Tools: Revit · Toposurface · Building Pad · Section Box

Files in this lab

LittleHouse.rvt  (open in Autodesk Revit)

Step-by-step

Open the site plan view of the project.

Project Browser ▸ Floor Plans ▸ Site

Start the Toposurface tool and place boundary points at elevation 0.

Massing & Site ▸ Model Site ▸ Toposurface ▸ Place Point ▸ Elevation 0

Raise the far points: change the elevation on the Options Bar as you place them.

Options Bar ▸ Elevation 900 ▸ click to place the back points

Assign a grass material to the surface, then finish it.

Properties ▸ Material ▸ Grass ▸ Finish Surface

Sketch a building pad over the foundation footprint on the lower level.

Massing & Site ▸ Model Site ▸ Building Pad ▸ Pick Walls ▸ Finish

Frame the result in 3D with a section box to see the pad cutting the terrain.

3D view ▸ Properties ▸ Section Box ✓ ▸ drag the handles

Test it

The 3D view shows the terrain with a grass material, and the building pad cuts a level recess into the toposurface at the foundation walls.

© 2026 Tertiary Infotech Academy Pte Ltd · UEN: 201200696W