Lab 3 — Create an In-Place Mass and Apply Materials

Autodesk Revit Architecture Masterclass (C508) · Topic 01: Design Modeling in Revit · Tertiary Infotech Academy Pte Ltd

Goal

Model a conceptual building volume as an in-place mass, then use the Materials browser to control how model elements display in shaded and rendered views.

You'll build

An in-place conceptual mass with materials applied by category and by face.

Tools: Revit · In-Place Mass · Materials Browser · Paint tool

Files in this lab

ModelCreation.rvt  (open in Autodesk Revit)

Step-by-step

Start an in-place mass and name the new mass family.

Massing & Site ▸ Conceptual Mass ▸ In-Place Mass ▸ name it Tower ▸ OK

Draw a closed profile and create a form from it.

Draw ▸ Rectangle ▸ select the profile ▸ Modify ▸ Create Form ▸ Solid Form

Finish the mass to return to the project environment.

In-Place Editor ▸ Finish Mass

Open the Materials browser and duplicate a material to customise it.

Manage ▸ Settings ▸ Materials ▸ right-click Concrete ▸ Duplicate

Set the material's graphics: shading colour, surface pattern and cut pattern.

Material Editor ▸ Graphics ▸ Shading ▸ choose a colour ▸ OK

Paint the new material onto one face of the mass.

Modify ▸ Geometry ▸ Paint ▸ pick the material ▸ click a face

Test it

The mass displays the painted material on the selected face in a shaded 3D view, and the duplicated material appears in the project's material library.

© 2026 Tertiary Infotech Academy Pte Ltd · UEN: 201200696W