Lab 8 — Create Stairs and Railings

Autodesk Revit Architecture Masterclass (C508) · Topic 02: Part Lists for Modeling in Revit · Tertiary Infotech Academy Pte Ltd

Goal

Complete the interior by adding a staircase from the store floor to the mezzanine, then modify the mezzanine railing and change its type to a pipe railing.

You'll build

A staircase connecting the store floor to the mezzanine, with a matching pipe railing on the mezzanine edge.

Tools: Revit · Stair tool · Railing tool

Files in this lab

LittleHouse.rvt  (open in Autodesk Revit)

Step-by-step

Open the lower-level plan and start the Stair tool.

Architecture ▸ Circulation ▸ Stair

Set the base and top levels for the run in the Properties palette.

Properties ▸ Base Level: 01 ▸ Top Level: Mezzanine

Choose the railing type to generate with the stair.

Modify | Create Stair ▸ Tools ▸ Railing ▸ pick a type

Draw the run: click the start point, move up-screen, click the end point.

Components ▸ Run ▸ Straight ▸ click start ▸ click end

Finish the stair and inspect it in 3D.

✓ Finish Edit Mode ▸ Default 3D View

Select the mezzanine railing and change it to a pipe railing type.

Select railing ▸ Type Selector ▸ Railing: Pipe

Test it

The 3D view shows a complete stair with railings from the store floor to the mezzanine, and the mezzanine edge railing is the pipe type.

© 2026 Tertiary Infotech Academy Pte Ltd · UEN: 201200696W